First, with a standard MIDI cable, connect the MIDI Out of the TD-15 to the MIDI In on the back panel of the TR-8.

Once you've made this connection, turn off the "Standard MIDI Setup Lock" in the TD-15. Here's how:

- On the TD-15, press the MENU button.

- Press the down cursor button until the "SYSTEM" icon appears in the display.

- Press the function 3 [F3] button (SYSTEM).

- Press cursor down to choose "MIDI."

- Press F3 (ENTER).

- Cursor to "Std MIDI Setup Lock."

- Turn the dial to choose "OFF."

- Press EXIT three times to return to the main kit menu.

Next, temporarily turn down the volume on your TD-15, but keep the volume up on your TR-8. Since you'll be triggering the sounds in the TR-8 from the TD-15's pads, you'll want to have its volume turned up. You'll also want to choose your favorite kit in the TR-8. By default, you may notice that you'll already be triggering a few of the TR-8's sounds when you play on the TD-15's pads (kick, snare, hi-hat, etc). Cool!!!!

Follow these steps if you want to trigger specific instruments that are assigned to the TR-8's pads from the TD-15's pads:

- On the TD-15, press MENU.

- Press the down cursor button once so that the "MIDI" icon appears in the left of the display.

- Press F1(MIDI).

- Hit one of the pads on the TD-15 (head or rim).

- Turn the dial to select the Note Number for the pad.

  - You can choose any of the following assigned note numbers in the TR-8:

    - BASS DRUM = 36

    - SNARE DRUM = 38

    - LOW TOM = 43

    - MID TOM = 47

    - HIGH TOM = 50

    - RIM SHOT = 37

    - HAND CLAP = 39

    - CLOSED HI-HAT = 42

    - OPEN HI-HAT = 46

    - CRASH CYMBAL = 49

    - RIDE CYMBAL = 51

- Repeat the process for any of the remaining TD-15 pads if desired or needed.

- When you're finished, and you are triggering the sounds like you want them, press EXIT twice on the TD-15. The new changes are saved in the Kit automatically.

Note: You'll need to repeat the assignment process in each Kit of the TD-15.